Baby Cry Detection

Using Voice Kit - RA6E1

Description

This document covers the development and testing of the Baby Cry Detection demo using Renesas’s VK - RA6E1 kit & Reality AI Tools - explorer tier. The portal is useful to generate AI models for non-visual sensing applications.

This document contains the following sections:

| Lab Section | Demo Steps |
| --- | --- |
| Section 1 | Prerequisites |
| Section 2 | Data Collection |
| Section 3 | Model Generation |
| Section 4 | Deploying & testing the Models |
| Section 5 | Appendix |

Sections should be followed sequentially.

| Lab Objectives - Gain hands-on experience on developing a simple AI demo using Renesas’s VK-RA6E1 kit + Reality AI Tools (using our explorer tier subscription). | Lab Materials - Please verify you have the following materials at your lab station. - PC with Windows 11. - Project files provided by Renesas – AI COE team. - E2-Studio IDE v5.6 (link in section 1 of the document). - Reality AI Tools – Explorer Tier Account |
| --- | --- |

Lab Sections

1	Prerequisites	2

2	Data Collection	7

3	Creating Models	17

4	Deploying the model	28

5	Appendix	32

# Prerequisites

Overview

This section covers the prerequisites for running the baby cry detector.

Procedural Steps

|  | This lab requires the VK – RA6E1 kit. The kit comes with: - RA6E1 board. - Necessary USB cables. Kit Link: TW001-VUIA6E1POCZ - RA6E1 Voice User Demonstration Kit ¦ Renesas |
| --- | --- |
|  | This lab requires Renesas E2-Studio IDE 2024-07 or newer with FSP 5.6. Platform installer available here: Link Take note of where e2studio is installed. |
|  | This lab requires an account on the Reality AI website. Login to Reality Tools (https://portal.reality.ai/login) using the username and password provided. Reach out to Renesas AI COE Customer Success team members (rai-cs@dm.renesas.com) if you need credentials + instruction documents for the portal. Leave the browser open in the background. |
|  | You have finished this section. |

# Data Collection

Overview

This section covers collecting data with the Reality-AI eclipse plugin using e2studios.

Procedural Steps

|  | Open E2 Studio IDE and select a workspace. Click Launch. |
| --- | --- |
|  | Go to File à Import... Then choose General->Existing Projects into Workspace Click Next to continue. |
|  | Click Select root directory and click Browse to go to the location of the example project folder. The demo project is imported into the workspace |
|  | Return to the Reality-AI tool and start a new project. Click on +Add Project, give the project a name and, optionally, a description. Then click the +Add Project button in the lower right. Note: If you are using Explorer Tier Subscription, skip creating a new project and just use the current Baby Cry Detection Project to upload data and run analysis. |
|  | Navigate to the username on the top-right of the screen and select API Key Option. |
|  | Copy this API key to the clipboard. This key will be used to connect e2 studio IDE and Reality AI Tools. |
|  | Open e2 studio and navigate to Renesas AI --> Reality AI Authentication (note: If Renesas AI does not appear in the menu, consult the appendix) Click on Add to add an API key and connect the account to your workspace. Paste the API key and click Apply and Close The IDE can now connect to the Reality AI site |
|  | Navigate to Renesas AI --> Show View --> Reality AI Data Storage Tool A new window will open in the bottom pane. Click-and-drag the view to the main view area by clicking on the tab and drag-drop in the tab area of the main view. Select the project in the pulldown, then click on “Data Connection" button. |
|  | In the menu that pops up, select the COM port, baudrate (921600), and communication protocol parameters (8-N-1). Then press Connect --> then press Close. |
|  | Once connected, the status (top left) should show “Receiving data...” with a check mark. Note: the green check mark indicates that the connection is successful. Not that it is receiving data yet. |
|  | Build the project by clicking the hammer icon The project should build without error. |
|  | Start a debug session by clicking the Debug Icon in the toolbar Click “Allow access” if you get a warning from Windows Defender. Check the “Remember my decision” checkbox and click Switch if the following dialog appears. |
|  | Click the Resume button twice. The program is now running |
|  | Click on the “Reality AI Data Storage Tool” view and verify the presence of a wave form in the Signal view. If there is nothing showing, click File review and then Live view (highlighted below). If there is still no waveform, recheck the prerequisite steps to make sure the connections are correct. |
|  | You can enter a name in the Class Name text box and capture any length of data. Note: It is recommended that you have the Unlimited checkbox selected. This will enable you to record variable length of data. |
|  | There are two methods to collect data: Number of frames per file and Unlimited. If the user selects Unlimited (by checking the Unlimited checkbox) data collection will occur until the user manually stops the collection process. Otherwise (Unlimited box unchecked) the collection will automatically stop once the Number of frames per file is reached. For this project, setting the Number of frames per file to 200 results in approximately 52 seconds of data. The remainder of this lab assumes Unlimited capture |
|  | In the Reality AI Data Storage Tool, check the Unlimited checkbox. Enter the Class name baby_cry. We have provided some Baby Cry audio files. Play them using your phone or PC speaker while the voice kit records the audio. Once Baby Cry recordings are finished, start a new recording by entering the class name as everything_else. Here you can record the audio while just sitting quietly, talking or typing on the keyboard. Record it for 2-3 minutes. Goal is to capture the surrounding environment. |
|  | This concludes the data capture portion. Click the red square icon to terminate the debug session. (If you do not see the terminate icon, select the Debug perspective in the upper right corner) |
|  | Collected data will show up in the list of metadata/ file description below. You can now upload the data to Reality AI Tools using Upload and run analysis on it as usual. Follow these steps to upload the data files collected in the last section: - Select the files using click + ctrl-click - Scroll the view to expose the Upload... button - Click the Upload... button |
|  | A dialog box should appear with the projects currently defined on the Reality AI website. Select the Baby Cry Demo project. Set the Sampling rate to 4096 and press OK The data files, along with the metadata.csv file will be uploaded, and a confirmation dialog should appear. |
|  | You have finished this section. |

# Creating Models

Overview

This section covers creating AI models on the collected dataset. A Reality AI Tools explorer tier account is required. Reach out to Renesas AI COE Customer Success team members (rai-cs@dm.renesas.com) if you need credentials + instruction documents for the portal.

Procedural Steps

|  | If not already logged in to Reaity AI Tools, use this link: https://portal.reality.ai/login with the provided password. Reach out to Renesas – Reality AI team members for credentials + instructions documents for the portal if needed. |
| --- | --- |
|  | Format the data. Click on the Baby Cry Demo project to set active. Click on the Data tab on the left to expand and follow these steps: - Click on Curate. - Click on Source Files to expand. - Check box next to File Name to select all files - Click on the Action button. - Click on Format Selected This opens a dialog box. |
|  | Follow these steps: - Click on Data to expand the drop down - Use the scroll bar to scroll down (or mouse wheel) Select the second class as “Target Class” from the drop-down menu. Assign sample rate as 4096. Click Confirm. |
|  | Verify that the data shape is now 1 X {num of samples} and the Sample rate is 4096 Hz |
|  | Preprocess/segment the data files. Click the Action button and select *** Segment List from Selected |
|  | Why Segmentation? One of the main purposes of generating models from Tools is to deploy them to a variety of Renesas MCU’s. To that effect, these models must process live data within a resource constrained environment. So, for practical purposes, a model might be looking at 1 second, 500 ms, or even a smaller window length to make quick predictions on Realtime data. As opposed to a few seconds (or minutes or hours) long data stream. To mimic that effect, we break down the raw training data and feed that to the model generation engine to start learning what it is going to see in a live (production) setting. |
|  | Assign window length, overlap, and provide name to the list. Follow these steps: - Set Window Length to 4096 - Click the 50% button to select a 50% offset - Give the list a meaningful name - Click Submit The number of estimated samples will be dependent on the amount of data collected. So if your estimated samples don’t match the below screenshot, its still okay. Window Length: The window length determines how much data will be considered by the AI to decide on a classification. Offset: The offset determines how far ahead from the start of the last window the parser moves before creating a new window. General Guide: 50% Overlap is usually a good compromise between covering starting-point variations in the data and too much redundancy. Use non-overlapping windows when you have a great deal of data, with longer offsets. Typically, users will do initial exploration and training on a subset of the available data. Use All Shifts (offset = 1) for testing after you have a suitable candidate classifier, and you want to simulate performance on a stream of new data arbitrarily sampled. |
|  | After successfully creating the segmented list, navigate to AI Explore à Classes |
|  | Reality AI Tools has options for creating 3 types of AI Models: Classes: When there is labeled categorical data (option used in the tutorial). You might have noticed that we uploaded perfectly labeled data in section 1. This is needed for classification models as supervised learning is being performed. Values: When discrete int or float values are used instead of categories. (Example: What is the exact temperature of a machine? Or What is the exact value of tire pressure of a car?). This is also supervised learning. Anomalies: This is an Anomaly Detection module. It is a semi-supervised model where the user only needs examples of Normal data to create a baseline model. |
|  | Click on the newly created list and then click on “Start Exploring” to start the feature discovery and model training process. |
|  | Once the explore finishes, select the highest performing model (we recommend model with feature space: Spectral Magnitude) and click on the Create Base Tool button. Hover over each Explanation to get more information about the model. |
|  | What is happening in the background: Clicking on the Start Explore button will turn the Reality AI algorithm loose on your data. It will create several optimized feature sets and machine learning models that best fit the classification problem and then summarize each model’s accuracy and resource consumption. What is the algorithm doing: AI Explore first creates a balanced subset of the sample list (meaning a list in which each class has equal numbers). If the sample list is exceptionally large, it may also sub-sample for a shorter processing time. Explore then runs through a procedure in which the Reality AI algorithms attempt to discover the best possible set of features and machine learning parameters for separating the different training classes represented in the data. The feature sets that are most promising are then used to construct machine learning models, which are trained on the sub-sample and put through a K-Fold validation. Only the best performing results are displayed. Several hundred different feature sets and machine learning models are compared in a typical AI Explore run. What is K-Fold validation: K-Fold is where a given data set is split into a K number of sections/folds where each fold is used as a testing set at some point. Let us take the scenario of 10-Fold cross validation(K=10). Here, the data set is split into 10 folds. In the first iteration, the first fold is used to test the model, and the rest are used to train the model. In the second iteration, the 2nd fold is used as the testing set while the rest serves as the training set. This process is repeated until each fold of the 10 folds has been used as the testing set. We use K=10 in the AI Explore page. |
|  | Provide a name or use the one suggested. Click Add. The icon will change, indicating the base tool has been created. |
|  | Now that the model is ready to be deployed, Follow these steps to produce a new package. - Click on Deploy->Embedded. - Click on the Trained Tool Description list - Click on + New Package |
|  | Set the options as indicated. The deployed name could be baby_cry_alp_v1 to match the code in e2studios. Choosing another name will require additional edits to the code. The package will take a few minutes to generate. |
|  | It will take ~10-15 minutes for the model to be available for download. Once ready, download the zip file using the highlighted button. |
|  | There are 9 files in the downloaded archive. Copy and paste all files to the src directory of the E2-Studio project workspace (src --> realityai), overwriting the current files. Note: file names may be different depending on the project name assigned in tools. In the files below, the model function call is specified in baby_cry_alp_v1_model.h. |
|  | You have finished this section. |

# Deploying & testing the model

Overview

This section covers testing the model on hardware.

Procedural Steps

|  | Open hal_entry.c and import the header file. If the deployed model name was baby_cry_alp_v1 then no changes are required. Otherwise, edit line# 37 to the deployed model’s header file. |
| --- | --- |
|  | Change line# 286, 294 & 302 from baby_cry to whatever the class name you assigned for Baby Cry event. |
|  | Build & debug the project as before. Click on the Resume button twice. Now the board is running on inference mode. |
|  | To check the model accuracy and predictions, navigate to Renesas AI – Show View – Reality AI Live Monitor (make sure you disconnect the Data Storage Tool connection first). |
|  | Click on Data Connection and again set the Baud rate to 921600 and appropriate COM port. |
|  | Once connected, you should start to see the results of live model predictions. In the above image, 1 represents baby_cry class and 2 represents everything_else classes. Dominant class appears higher on the class score (y-axis). Play a few Baby Cry audios from YouTube. You should see 1 appear on x-axis and blue bars appear higher on y-axis. |
|  | What should I do if model is not performing well? Collect some more data and retrain the model. Usually additional data collection helps in creating better performing models across different conditions. Double check if the data collection method and testing method are the same. Collecting more data across these variations should help. |
|  | To validate the model on the board for the already collected data, go to Renesas AI and navigate to Hardware In-Loop Test option. Select the inference package/model. Here you will need to select the trained model from your Reality AI Tools account. If you have Explorer Tier access, select the baby_cry_alp_v1 (pre-loaded model). |
|  | Double check to see the board is connected and select the right COM port. Click Start test. The plugin will automatically generate a project on your drive which will be used to test the model on the hardware. |
|  | ## The plugin will also download the simulation data (part of training data of the model) and use it to test on the hardware. Once started, you should start seeing Inference Time and Overall Accuracy on the screen. |
|  | You have finished this section. |

# Appendix

Overview

If the Reality AI items do not appear in the title bar perform the following steps.

Procedural Steps

|  | Click on Help -> Install Renesas IDE Features... |
| --- | --- |
|  | Select the Reality-AI features and click Finish. Restart e2studios when prompted. |
|  | You have finished this section. |

END OF LAB

THANK YOU