Asset Movement Recognition workshop

Using Renesas RL78 - G23FPB

Description

The application used in this lab is built to run on RL78-G23 FPB Solution Kit. A foundation Asset Movement Recognition (AMR) project will be created from scratch and populated with several HAL drivers provided by the smart configurator.

This document covers the development and testing of the Asset Tracking demo using Renesas’s RL78-G23 FPB & Reality AI Tools portal. The portal is useful to generate AI models for non-visual sensing applications.

This document contains the following sections:

| Lab Section | Demo Steps |
| --- | --- |
| Section 1 | Prerequisites |
| Section 2 | Accelerometer enablement |
| Section 3 | Data Collection |
| Section 4 | Creating Models |
| Section 5 | Deploying the Models |
| Section 6 | Appendix |

Sections should be followed sequentially.

| Lab Objectives - Gain hands-on experience on developing a simple AI demo using Renesas RL78-G23 FPB + PMOD accelerometer. | Lab Materials - Please verify you have the following materials at your lab station. - PC - Project files provided by Renesas - Reality AI team (link in section 1 of the document). - E2-Studio IDE v2025-04 or later - link in section 1 of the document). - Reality AI Tools Account |
| --- | --- |

Lab Sections

1	Prerequisites	2

2	Accelerometer enablement	5

3	Data Collection	11

4	Creating Models	17

5	Deploying the model	26

6	Appendix	29

# Prerequisites

Overview

This section covers the prerequisites for running the asset movement recognition demo.

Procedural Steps

| ## 1.1 | This lab requires the RL78-G23 FPB and RenesasICM-426700P PMOD accelerometer. You will need: - RL78-G23 FPB board. - Renesas ICM-42670-P PMOD Board - Micro USB device cable (type-A male to micro-B male) |
| --- | --- |
| ## 1.2 | Assemble the kit as shown below. 1. Connect the PMOD board to the PMOD 1 connection on the board. 2. Connect the PC to Micro USB on the board. |
| ### 1.3 | This lab requires Renesas E2-Studio IDE 2025-04 or newer. Platform installer available here: Link. Take note of where e2studio is installed. |
| ## 1.4 | This lab requires an account on the Reality AI website. Login to Reality Tools (https://portal.reality.ai/login) using the username and password provided. Reach out to Renesas – Reality AI team members if you need credentials + instruction documents for the portal. Leave the browser open in the background. |

# Enabling the Accelerometer

Overview

The following section describes the steps required to set up an accelerometer demo project for AIKRA6M3 kit.

Procedural Steps

|  | ## Launch e2 studio from the Windows start menu or directly from the installation folder. If you have multiple versions of e2 studio installed, please make sure to launch the version of the FSP setup that appropriate for this project. The FSP migration guide found in renesas/aiot-ready (same as this document path) will help you to switch between FSP versions minimizing compatibility or other issues in the development environment. |
| --- | --- |
| ### 2.2 | ## In the Eclipse Launcher window, specify the destination for the new workspace. It is recommended to keep the path simple and avoid using spaces. |
| 2.3 | ## Click Launch to start e2 studio in the specified path. If prompted, press Apply to dismiss pop up window asking for permission to log and report usage (it will remain disabled). |
| 2.4 | ## The welcome screen will show inside the new workspace. It can be dismissed by clicking on the Hide button in the top-right corner. |
| 2.5 | ## Go to File -> Import --> Import Existing Project. Click Finish after importing. |
| 2.28 | ## The project is now ready to compile. Press the “hammer” icon to start building the project. |
| 2.29 | ## Once the build has finished, the Console pane in the lower-right corner of e2 studio will report zero error and warning. |
| 2.30 | ## The application is now ready to be programmed and run on the FPB kit. ## Verify that the accelerometer in PMOD 1. ## Press the “bug” icon to begin the debug session. Preferably right click to the launch file and start debugging from there. |
| 2.31 | ## Click the Resume button or press F8 on the keyboard to start the application. Press Resume or F8 again to resume the application and begin executing user code. |
| 2.32 | ## Click the Terminate button or press Ctrl + F2 on the keyboard to stop the application and terminate the debug session. |

# Data Collection

Overview

This section covers collecting data with the Reality-AI eclipse plugin using e2studios.

Procedural Steps

| ## 3.1 | Continue from previous steps |
| --- | --- |
| ## 3.2 | Connect RL78-G23 FPB Device USB Type C com port (J11) to the computer. |
| ## 3.3 | Return to the Reality-AI tool and start a new project. Click +Add Project to expand the project creation tab, give your new project a name and (option) description. Then click the + Add Project button in the lower right. |
| ## 3.4 | Navigate to the username on the top-right of the screen and select API Key Option. |
| ## 3.5 | Copy this API key to the clipboard. This key will be used to connect e2 studio IDE and Reality AI Tools. |
| ## 3.6 | Open e2 studio and navigate to Renesas AI --> Reality AI Authentication (note: If Renesas AI does not appear in the menu, consult the appendix) Add the Username and API key and click Apply and Close The IDE can now connect to the Reality AI site |
| ## 3.7 | From the main menu bar, navigate to Renesas AI --> Show View --> Reality AI Data Storage Tool A new window will open in the bottom pane. Click-and-drag the view to the main view area by clicking on the tab and drag-drop in the tab area of the main view. |
| ## 3.8 | Click on the Data file settings and verify CVS file and Output folder. The Output folder is relative to the project. Click OK to close the dialog. |
|  | Select the appropriate e2 studio project in the pulldown, then click on “Data Connection" button. |
| ## 3.9 | In the Connection settings window that pops up, select the USB and the appropriate COM Port. Then press Connect --> then press Close. |
| ## 3.10 | Click on the “Reality AI Data Storage Tool” view and verify the presence of a wave form in the Signal view. If there is nothing showing, click File review and then Live view (highlighted below). If there is still no waveform, recheck the prerequisite steps to make sure the connections are correct. You should see a green checkmark and “Receiving data…” indicator as well as a waveform on the Signal view window. |
| ## 3.11 | Note that the Data Storage tool reflects the data in the project’s Output folder. The preexisting data currently there was collected during the lab development and includes date and time information. New data will be collected in the following sections to supplement this. |
|  | There are two methods to collect data: Number of frames per file and Unlimited. If the user selects Unlimited (by checking the Unlimited checkbox) data collection will occur until the user manually stops the collection process. Otherwise (Unlimited box unchecked) the collection will automatically stop once the Number of frames per file is reached. For this project, setting the Number of frames per file to 200 results in approximately 52 seconds of data. The remainder of this lab assumes Unlimited capture. |
| ## 3.12 | You can now collect accelerometer data which can directly be used to train your Reality AI model. Here, you will collect data for a three class model for the idle, drop, and motion classes. In the Reality AI Data Storage Tool, check the Unlimited checkbox, enter the Class name “idle”, and then click Start new capture. Keep the board stationary for 60 seconds. After collecting a minute or so of idle data, click the Stop Capture button. You should see a new file appear within the data file table in the “Manage capture data” files tab with the idle class label. |
| ## 3.13 | For the “drop” class, you will repeatedly drop the board from ~7cm height onto your desk for 2 minutes. Be careful not to drop it so that the PMOD makes direct contact with the table as you may damage the connector. You may also want to place a mouse pad or similar surface beneath it as you collect the drop data. Update the class name to “drop”, hit Start new capture, and then perform the drops as data collects for 1-2 minutes. Click “Stop capture” once finished and verify you have a new data file with the drop class. You may see additional files other than the two that you just collected if you copied in the provided sample data. |
| ## 3.14 | For the “motion” class, you can slide the board across the table, as well as pick it up and move or rotate it through the air. Repeat the above data collection process (being sure to update the class name to motion, start new capture, stop capture) collecting 60 seconds of motion data. |
| ## 3.15 | You should now have the collected the idle, drop and motion files. Also, from copying the Output folder files (see step 2.24; ) there should be supplemental data which we collected previously for your convenience. Note that if you did not complete this step prior to performing data collection, you will need to manually merge the metadata.csv files from your data collection with the supplemental data metadata.csv Follow these steps to upload the data files collected in the last section: - Select the files using Control + Click (we recommend you include the supplemental data) - Locate the upload button on the bottom right; you may need to scroll down via the right hand slider. - Click the Upload button |
| ## 3.16 | A dialog box should appear with the projects currently defined on the Reality AI website. Select the appropriate project Set the Sampling rate to 1600 and press OK. The data files along with the metadata.csv file will upload, and a confirmation dialog should appear. |
|  | You have finished this section. |

# Creating Models

Overview

This section covers creating AI models on the collected dataset. A Reality AI Tools account is required.

Procedural Steps

| ## 4.1 | If not already logged in to Reaity AI Tools, use this link: https://portal.reality.ai/login with the provided password. Reach out to Renesas – Reality AI team members for credentials + instructions documents for the portal if needed. |
| --- | --- |
| ## 4.2 | First, we will format the data. Click on your current asset movement project to set it as active. You should see the tab expand as well as the Project indicator on the top ribbon update to reflect it. Then click on the Data tab on the left to expand it and then follow these steps: - Click on Curate. - Click on Source Files to expand. - Click the check box next to File Name to select all files. - Click on the Action button to open the dropdown. - Click on Format Selected to open the formatting dialog. |
|  | Follow these steps: - Click the first columns drop down button (default shows Data) - Use the scroll bar to scroll down (or mouse wheel) - Select Ignore to ignore this column of index values. - Click confirm to finish specifying the file format. |
| ## 4.3 | Verify that the data shape is now 3 X {num of samples} and the Sample rate is 1600Hz. |
| ## 4.4 | We will now preprocess the data into an appropriate format for the classifier. First, we will process the idle and motion data together, and then we will separately process the drop data as it requires an additional step to filter out the motion in-between drops. Select all of your idle and motion files (excluding drop files). Then click Action > Segment List From Selected. |
|  | Why Segmentation? One of the main purposes of generating models from Tools is to deploy them to a variety of Renesas MCU’s. To that effect, these models must process live data within a resource constrained environment. So for practical purposes, a model be looking at 1 second, 500 ms, or even a smaller window length to make rapid predictions on real-time data. This improves the response rate and minimizes memory and processing requirements in comparison to longer data streams. Segmenting the training data breaks down the raw training data into individual sample buffers which are provided to the AI Explore engine which processes the raw data packets into various feature spaces for building and training models. |
| ## 4.5 | For the idle and motion data, assign window length, overlap, and provide a name to the list. Follow these steps: - Set Window Length datapoints to 512. - For Offset, click on the “Non-Overlapping” button. This will automatically populate the Offset datapoints field so the data is processed into non-overlapping windows. - Give the list a meaningful name. The number of estimated samples will be dependent on the amount of data collected. So if your estimated samples don’t match the below screenshot, its still okay. Window Length: The window length determines how much data will be considered by the AI to decide on a classification. Offset: The offset determines how far ahead from the start of the last window the parser moves before creating a new window. General Guide: 50% Overlap is usually a good compromise between covering starting-point variations in the data and too much redundancy. Use non-overlapping windows when you have a great deal of data, with longer offsets. Typically, users will do initial exploration and training on a subset of the available data. The number of estimated samples will be dependent on the amount of data collected. So if your estimated samples don’t match the below screenshot, its still okay. Window Length: The window length determines how much data will be considered by the AI to decide on a classification. Offset: The offset determines how far ahead from the start of the last window the parser moves before creating a new window. General Guide: 50% Overlap is usually a good compromise between covering starting-point variations in the data and too much redundancy. Use non-overlapping windows when you have a great deal of data, with longer offsets. Typically, users will do initial exploration and training on a subset of the available data. - Click Submit. After a moment, you should see new sample list under the Data Sample Lists window. You may need to refresh the page if it does not appear after a few moments. |
|  | For the drop data, it is important that you only segment windows in which the device was dropped (i.e. falling and impact with the table, ignoring when you were picking up the kit and it was idle on the table) so the model can successfully learn the difference between general motion and a drop event. To do this, we can use our energy triggered segmentation tool to parse out key segments from the data stream. This allows you to specify data window start indices using an energy trigger function, allowing us to filter out low energy motion between successive drops. - Still in the Data > Curate tab, select the drop file(s) in the Source Files window, then click Action -> Segment List From Selected - Click the drop down for the Segmentation Method and select Energy Triggered. - For the Trigger Channel, select Magnitude. - Press Show Preview. This will demonstrate how your energy trigger parameters are performing with your data. - For Trigger Mode, use Amplitude with a Threshold of 25. - Set Window Length to 512 - For Capture Options, set Pre-Trigger to 120 and Min Separation to 1600 - Hit Refresh Preview. Verify that these settings are appropriate and you see the drop impact spikes being identified on the Trigger Channel and you can see the drop event highlighted in blue across the channels of data. Depending on how you executed your drops, you may need to adjust some of the Threshold or Min Separation. See below for a successful segmentation. - You may need to zoom in to better visualize your segmentation like above where only the first 32000 (20 seconds) datapoints are shown. Adjust your energy trigger parameters until you only have drop events captured and the motion between drops is ignored. Once you are satisfied, press the Continue button at the buttom of the energy segmentation page. - Give the Output Sample List meaningful name and press Submit. |
| ## 4.6 | Within Data > Curate go to Data Sample Lists and use Control + Click (or enable Multi-view) to select your idle/motion sample list and newly created drop sample lists. Then click Action > Merge Selected to New. Give the new list a descriptive name and press Create. You should see a confirmation that your merged list is created. You should now have three Data Sample Lists – your original idle/motion sample list, your energy triggered drop list, and the merged list which combines the former two. |
|  | Reality AI Tools has options for creating 3 types of AI Models: Classes: Produces models which classify labeled categorical data (like in this tutorial). This form of supervised machine learning requires explicitly labeled categorical data. Values: Produces models that compute a continuous value output (like regression) such as an int or a float instead of categorical output model . (Example: What is the exact temperature of a machine? Or What is the exact value of tire pressure of a car?). This is also supervised learning. Anomalies: This is an Anomaly Detection module. It is a semi-supervised model where the user only needs examples of Normal data to create a baseline model and then the model is tuned to be sensitive to specific degrees of anomalous data. |
| ## 4.7 | Navigate to the AI Explore > Classes page. AI Explore is where you can automatically process your data sample list into hundreds of feature spaces, train various machine learning models, and compare candidate model accuracy, resource requirement, and feature space explanations. Click on the newly created list and then click on “Start Exploring” to start the feature discovery and model training process. You will see an “In progress” indicator. After a few minutes of startup time, you will be able to view individual models as they successively complete. It will take approximately 5-20 minutes (depending on server load) for all models and feature spaces to complete and for the exploration finish. |
|  | What is happening in the background: Clicking on the Start Explore button begins an exploration of your dataset. Hundreds of optimized features and machine learning models are assessed to see which best fits your classification problem and then summaries of each model’s accuracy and resource requirements are provided. What is the algorithm doing: AI Explore first creates a balanced subset* of the sample list (meaning a list in which each class has equal numbers). If the sample list is exceptionally large, it may also sub-sample for a shorter processing time. Explore then runs through a procedure in which the Reality AI algorithms identify the best possible set of features and machine learning parameters for separating the different training classes represented in the data. The most promising feature sets and models put through a K-Fold validation, and the best performing results are displayed. Several hundred different feature sets and machine learning models are compared in a typical AI Explore run. What is K-Fold validation: K-Fold is where a given data set is split into k-number of groups or “folds” where each fold is used as a testing set at some point. Generally, Reality AI uses 10-fold cross validation (K=10). The data set is split into 10 folds. In the first iteration, 9 folds are used to train a given model, a 10th fold is held out from training, and the model is tested on the unseen fold. In the second iteration, a different individual fold is held out, and this new group of 9 remaining folds is used for training the model before it is again tested on the single hold out fold. This process is repeated until each of the 10 folds has been individually held out and used as a test set, and the reported k-fold accuracy is the aggregate performance of those 10 models their respective hold out data. We use K=10 in AI Explore. *note that in certain situations, an imbalanced dataset may be necessary or advisable, so a user can always use the Build > Train section of Tools to retrain a Base Tool using their entire (imbalanced) dataset |
| ## 4.8 | Once the explore finishes, you will be presented with several candidate models. Note that AI Explore will try and highlight some of the best candidate models with a crown, but for easier classification models such as this this AMR project you may often find better options further down in the list. For example, you can see the the 4th and 6thmodels model shown in the screenshot below achieve near perfect accuracy while requiring significantly less resources than the first three options. |
| ## 4.9 | Provide a name or use the one suggested, then click Add. The icon will change, indicating the base tool has been created. |
|  | Now that the model is ready to be deployed, Follow these steps to produce a new package. - Click on Deploy->Embedded. - Click on the Trained Tool Description list - Click on + New Package |
| ## 4.10 | For the deploy name, use “AIModel” in order to match the provided code from Section 2. Choosing another name will require additional edits to the code. Click Generate New Package after setting the options. It may take 5-15 minutes for the model to be available. You can use int16 as input data type. Select the toolchain CCRL 1.15 for the model export. |
| ## 4.11 | Once ready, download the zip file using the button on the right. You may need to refresh the page. |
|  | There are 7 files in the downloaded archive. Copy and paste all files except example_classifier.c & readme.txt to the src directory of the E2-Studio project workspace (src\realityai\), overwriting the current files. |
| ## 4.12 | You have finished this section. |

# Deploying the model

Overview

This section covers testing the model on hardware.

Procedural Steps

| ## 5.1 | Continue from previous steps |
| --- | --- |
| ## 5.5 | What should I do if model is not performing well? Collect some more data and retrain the model. Usually additional data collection helps in creating better performing models across different conditions. Double check if the data collection method and testing method are the same. Collecting more data across different variations should help. |
| ## 5.8 | ## The project is now ready to be compiled. Press the “hammer” icon to start building the project. |
| ## 5.9 | ## The application is now ready to be programmed and run on the AIK kit. Press the “bug” icon to begin the debug session. Preferably, right click on the launch file to start debugging from “Debug As”. |
| ## 5.10 | ## Click the Resume button or press F8 on the keyboard to start the application. Press Resume or F8 again to resume the application and begin executing user code. |
| ## 5.11 | Move the board as below and observe the LED indications No; Movement; LED Indicator. 1; Drop; Single LED ON. 2; Idle; Double LED ON. 3; Motion; Double LED Blink. |
| ## 5.12 | You have finished this section. |

# Appendix

Overview

If the Reality AI items do not appear in the title bar perform the following steps.

Procedural Steps

| ## 6.1 | Click on Help -> Install Renesas IDE Features... |
| --- | --- |
| ## 6.2 | Select the Reality-AI features and click Finish. Restart e2studios when prompted. |
| ## 6.3 | After Installing Reality AI Monitor open the window and go to settings, select the project created in previous section. In inference Module choose the project created in Reality AI Tools. Go to data connection and connect to the board. With Reality AI Monitor user has the option to view in real time the inference output of the workshop. |
|  | You have finished this section. |

END OF LAB

THANK YOU