Car State Detection

Using Renesas AIK-RA6M3

Description

The application used in this lab is built to run on AIK-RA6M3 Solution Kit. A foundation Car State Detection project is provided and already populated with several HAL drivers provided by the Flexible Software Package (FSP)

This document covers the development and testing of the Car State demo using Renesas’s AIK-RA6M3 & Reality AI Tools portal. The portal is useful to generate AI models for non-visual sensing applications.

This document contains the following sections:

| Lab Section | Demo Steps |
| --- | --- |
| Section 1 | Prerequisites |
| Section 2 | Accelerometer enablement |
| Section 3 | Data Collection |
| Section 4 | Creating Models |
| Section 5 | Deploying the Models |
| Section 6 | Appendix |

Sections should be followed sequentially.

| Lab Objectives - Gain hands-on experience on developing a simple AI demo using Renesas AIK-RA6M3 + PMOD accelerometer. | Lab Materials - Please verify you have the following materials at your lab station. - PC with Windows 10 or 11. - Project files provided by Renesas - Reality AI team (link in section 1 of the document). - Latest E2-Studio IDE (2025-04 or later) - link in section 1 of the document). - Reality AI Tools Account. |
| --- | --- |

Lab Sections

1	Prerequisites	2

2	Accelerometer enablement	5

3	Data Collection	11

4	Creating Models	17

5	Deploying the model	26

6	Appendix	29

# Prerequisites

Overview

This section covers the prerequisites for running the asset movement recognition demo.

Procedural Steps

| ## 1.1 | This lab requires the AIK-RA6M3 and RenesasICM-426700P PMOD accelerometer. The AIK-RA6M3 kit comes with: - AIK-RA6M3 board. - OV2640 Camera module - Renesas ICM-42670-P PMOD Board - Display - Ethernet patch cable - Micro USB device cable (type-A male to micro-B male) - Micro USB device cable (type-A male to type-C male) Assemble the kit as shown in the next section. |
| --- | --- |
| ## 1.2 | Assemble the kit as shown below. 1. Connect the PMOD board to the PMOD 1 connection on the board and set S4 dip-switch to IIC 2. Connect the PC to Micro USB on the boards. 3. Connect the PC to USB type C port on the board. |
| ### 1.3 | This lab requires Renesas E2-Studio IDE 2025-04 or newer with FSP 5.5.0 or newer. The current version of this document is intended for FSP 5.5; additional steps may be necessary to be successful on newer versions. Platform installer available here: Link. Download the windoes installer (of the latest released version): Take note of where e2studio is installed. |
| ## 1.4 | This lab requires an account on the Reality AI website. Login to Reality Tools (https://portal.reality.ai/login) using the username and password provided. Reach out to Renesas – AI CS team members if you need credentials + instruction documents for the portal at rai-cs@dm.renesas.com. Leave the browser open in the background. |

# Data Collection

Overview

This section covers collecting data with the Reality-AI eclipse plugin using e2studios.

Procedural Steps

| ## 3.1 | Continue from previous steps. |
| --- | --- |
|  | Open E2 Studio IDE and select a workspace. Click Launch. |
| ## 3.2 | Go to File à Import... Then choose General->Existing Projects into Workspace Click Next to continue. |
| ## 3.3 | Click Select root directory and click Browse to go to the location of the example project folder. The demo project is imported into the workspace |
| ## 3.4 | Return to the Reality-AI tool and start a new project. Click on +Add Project, give the project a name and, optionally, a description. Then click the +Add Project button in the lower right. Note: If you are using Explorer Tier Subscription, skip creating a new project and just use the current Car State Detection Project to upload data and run analysis. |
| ## 3.5 | Navigate to the username on the top-right of the screen and select API Key Option. Copy this API key to the clipboard. This key will be used to connect e2 studio IDE and Reality AI Tools. |
| ## 3.6 | Open e2 studio and navigate to Renesas AI --> Reality AI Authentication (note: If Renesas AI does not appear in the menu, consult the appendix) Click on Add to add an API key and connect the account to your workspace. Paste the API key and click Apply and Close The IDE can now connect to the Reality AI site |
| ## 3.7 | From the main menu bar, navigate to Renesas AI --> Show View --> Reality AI Data Storage Tool A new window will open in the bottom pane. Click-and-drag the view to the main view area by clicking on the tab and drag-drop in the tab area of the main view. |
| ## 3.8 | Click on the Data file settings and verify CVS file and Output folder. The Output folder is relative to the project. Click OK to close the dialog. |
|  | Select the appropriate e2 studio project in the pulldown, then click on “Data Connection" button. |
| ## 3.9 | In the Connection settings window that pops up, select the USB and the appropriate COM Port. Then press Connect --> then press Close. |
| ## 3.10 | Click on the “Reality AI Data Storage Tool” view and verify the presence of a wave form in the Signal view. If there is nothing showing, click File review and then Live view (highlighted below). If there is still no waveform, recheck the prerequisite steps to make sure the connections are correct. You should see a green checkmark and “Receiving data…” indicator as well as a waveform on the Signal view window. |
| ## 3.11 | Note that the Data Storage tool reflects the data in the project’s Output folder. The preexisting data currently there was collected during the lab development and includes date and time information. New data will be collected in the following sections to supplement this. |
|  | There are two methods to collect data: Number of frames per file and Unlimited. If the user selects Unlimited (by checking the Unlimited checkbox) data collection will occur until the user manually stops the collection process. Otherwise (Unlimited box unchecked) the collection will automatically stop once the Number of frames per file is reached. For this project, setting the Number of frames per file to 200 results in approximately 52 seconds of data. The remainder of this lab assumes Unlimited capture. |
| ## 3.12 | You can now collect accelerometer data which can directly be used to train your Reality AI model. Here, you will collect data for a three class model for the idle/ car_off, car_on_with_ac & driving. In the Reality AI Data Storage Tool, check the Unlimited checkbox, enter the Class name “idle” or “car_off”, and then click Start new capture. Keep the board stationery for 3 minutes in the car by placing it on the dashboard. We recommend using some form of adhesive tape to make sure it doesn’t move. Movement will affect the accuracy of models. After collecting a minute or so of idle data, click the Stop Capture button. You should see a new file appear within the data file table in the “Manage capture data” files tab with the idle class label. |
| ## 3.13 | For the “car_on_with_ac”, just switch on the car and turn on the ac to 50% and re-do the process of data collection for 3 minutes. |
| ## 3.14 | For the “driving” class, start the data collection and drive around the car for 3 minutes. |
| ## 3.15 | You should now have the collected the 3 class data files. Also, from copying the Output folder files (see step 2.24; ) there should be supplemental data which we collected previously for your convenience. Note that if you did not complete this step prior to performing data collection, you will need to manually merge the metadata.csv files from your data collection with the supplemental data metadata.csv . Follow these steps to upload the data files collected in the last section: - Select the files using Control + Click (we recommend you include the supplemental data) - Locate the upload button on the bottom right; you may need to scroll down via the right hand slider. - Click the Upload button |
| ## 3.16 | A dialog box should appear with the projects currently defined on the Reality AI website. Select the appropriate project Set the Sampling rate to 1600 and press OK. The data files along with the metadata.csv file will upload, and a confirmation dialog should appear. |
|  | You have finished this section. |

# Creating Models

Overview

This section covers creating AI models on the collected dataset. A Reality AI Tools account is required.

Procedural Steps

| ## 4.1 | If not already logged in to Reaity AI Tools, use this link: https://portal.reality.ai/login with the provided password. Reach out to Renesas – Reality AI team members for credentials + instructions documents for the portal if needed. |
| --- | --- |
| ## 4.2 | First, we will format the data. Click on your current asset movement project to set it as active. You should see the tab expand as well as the Project indicator on the top ribbon update to reflect it. Then click on the Data tab on the left to expand it and then follow these steps: - Click on Curate. - Click on Source Files to expand. - Click the check box next to File Name to select all files. - Click on the Action button to open the dropdown. - Click on Format Selected to open the formatting dialog. |
|  | Follow these steps: - Click the first columns drop down button (default shows Data) - Use the scroll bar to scroll down (or mouse wheel) - Select Ignore to ignore this column of index values. - Click confirm to finish specifying the file format. |
| ## 4.3 | Verify that the data shape is now 1 X {num of samples} and the Sample rate is 1600Hz. |
| ## 4.4 | We will now preprocess the data into an appropriate format for the classifier. Select all of your files. Then click Action > Segment List From Selected. |
|  | Why Segmentation? One of the main purposes of generating models from Tools is to deploy them to a variety of Renesas MCU’s. To that effect, these models must process live data within a resource constrained environment. So, for practical purposes, a model be looking at 1 second, 500 ms, or even a smaller window length to make rapid predictions on real-time data. This improves the response rate and minimizes memory and processing requirements in comparison to longer data streams. Segmenting the training data breaks down the raw training data into individual sample buffers which are provided to the AI Explore engine which processes the raw data packets into various feature spaces for building and training models. |
| ## 4.5 | For the idle and motion data, assign window length, overlap, and provide a name to the list. Follow these steps: - Set Window Length datapoints to 512. - For Offset, click on the “Non-Overlapping” button. This will automatically populate the Offset datapoints field so the data is processed into non-overlapping windows. - Give the list a meaningful name. - Click Submit. After a moment, you should see new sample list under the Data Sample Lists window. You may need to refresh the page if it does not appear after a few moments. |
|  | Reality AI Tools has options for creating 3 types of AI Models: Classes: Produces models which classify labeled categorical data (like in this tutorial). This form of supervised machine learning requires explicitly labeled categorical data. Values: Produces models that compute a continuous value output (like regression) such as an int or a float instead of categorical output model . (Example: What is the exact temperature of a machine? Or What is the exact value of tire pressure of a car?). This is also supervised learning. Anomalies: This is an Anomaly Detection module. It is a semi-supervised model where the user only needs examples of Normal data to create a baseline model and then the model is tuned to be sensitive to specific degrees of anomalous data. |
| ## 4.7 | Navigate to the AI Explore > Classes page. AI Explore is where you can automatically process your data sample list into hundreds of feature spaces, train various machine learning models, and compare candidate model accuracy, resource requirement, and feature space explanations. Click on the newly created list and then click on “Start Exploring” to start the feature discovery and model training process. You will see an “In progress” indicator. After a few minutes of startup time, you will be able to view individual models as they successively complete. It will take approximately 5-20 minutes (depending on server load) for all models and feature spaces to complete and for the exploration finish. |
|  | What is happening in the background: Clicking on the Start Explore button begins an exploration of your dataset. Hundreds of optimized features and machine learning models are assessed to see which best fits your classification problem and then summaries of each model’s accuracy and resource requirements are provided. What is the algorithm doing: AI Explore first creates a balanced subset* of the sample list (meaning a list in which each class has equal numbers). If the sample list is exceptionally large, it may also sub-sample for a shorter processing time. Explore then runs through a procedure in which the Reality AI algorithms identify the best possible set of features and machine learning parameters for separating the different training classes represented in the data. The most promising feature sets and models put through a K-Fold validation, and the best performing results are displayed. Several hundred different feature sets and machine learning models are compared in a typical AI Explore run. What is K-Fold validation: K-Fold is where a given data set is split into k-number of groups or “folds” where each fold is used as a testing set at some point. Generally, Reality AI uses 10-fold cross validation (K=10). The data set is split into 10 folds. In the first iteration, 9 folds are used to train a given model, a 10th fold is held out from training, and the model is tested on the unseen fold. In the second iteration, a different individual fold is held out, and this new group of 9 remaining folds is used for training the model before it is again tested on the single hold out fold. This process is repeated until each of the 10 folds has been individually held out and used as a test set, and the reported k-fold accuracy is the aggregate performance of those 10 models their respective hold out data. We use K=10 in AI Explore. *note that in certain situations, an imbalanced dataset may be necessary or advisable, so a user can always use the Build > Train section of Tools to retrain a Base Tool using their entire (imbalanced) dataset |
| ## 4.8 | Once the explore finishes, you will be presented with several candidate models. Note that AI Explore will try and highlight some of the best candidate models with a crown. Check out the resource consumption and accuracy statistics and select an appropriate model. |
| ## 4.9 | Provide a name or use the one suggested, then click Add. The icon will change, indicating the base tool has been created. |
|  | Now that the model is ready to be deployed, Follow these steps to produce a new package. - Click on Deploy->Embedded. - Click on the Trained Tool Description list - Click on + New Package |
| ## 4.10 | Set the options as indicated. For the deploy name, use “car_state” in order to match the provided code from Section 2. Choosing another name will require additional edits to the code. Click Generate New Package after setting the options. It may take 5-15 minutes for the model to be available. |
| ## 4.11 | Once ready, download the zip file using the button on the right. You may need to refresh the page. |
|  | There are 7 files in the downloaded archive. Copy and paste all files except example_classifier.c & readme.txt to the src directory of the E2-Studio project workspace (src\realityai\), overwriting the current files. Alternatively, you can select the project in the Project Explorer navigate to Renesas AI -> Download inference module from Reality AI Tools and in the new window select the model you just deployed and click Download. |
| ## 4.12 | You have finished this section. |

# Deploying the model

Overview

This section covers testing the model on hardware.

Procedural Steps

| ## 5.1 | Continue from previous steps |
| --- | --- |
| ## 5.2 | Double click on “configuration.xml” and hit “Generate Project Content” |
| ## 5.3 | Open hal_entry.h and import the header file. If the deployed model name was car_state_model.h then no changes are required. Otherwise, edit the #include to the deployed model’s header file and change the following : #include "car_state_model.h" #define RAI_PREDICT(x) car_state_predict(x) #define RAI_GET_MDOEL() get_car_state_model() enum rai_aiclass{ rai_aiclass_no_results = car_state_no_results, rai_aiclass_off = car_state _car_off, rai_aiclass_on = car_state _car_on_with_ac, rai_aiclass_driving = car_state_motion, }; As seen below in hal_entry.h |
|  | What should I do if model is not performing well? Collect some more data and retrain the model. Usually additional data collection helps in creating better performing models across different conditions. Double check if the data collection method and testing method are the same. Collecting more data across different variations should help. |
| ## 5.8 | ## The project is now ready to be compiled. Press the “hammer” icon to start building the project. |
| ## 5.9 | ## The application is now ready to be programmed and run on the AIK kit. Press the “bug” icon to begin the debug session. Preferably, right click on the launch file to start debugging from “Debug As”. |
| ## 5.10 | ## Click the Resume button or press F8 on the keyboard to start the application. Press Resume or F8 again to resume the application and begin executing user code. |
| ## 5.11 | Test the model by performing the same actions you did during data collection. No; Movement; LED Indicator. 1; Car On with AC; Red LED blinks. 2; Car Off; Green LED on. 3; Driving; Green + Blue LED on. |
|  | You can also view the results of live testing using AI Live Monitor. Navigate to Renesas AI --> Show View --> Reality AI Live Monitor |
|  | Select the correct project from dropdown and click on Data Connection |
|  | Make sure the data storage tool is disconnected. Then select USB and the appropriate COM port. Click Connect. |
|  | You should be able to see the results of the model running live on the page. Below is an example screenshot from our testing of an example model. |
| ## 5.12 | You have finished this section. |

# Appendix

Overview

If the Reality AI items do not appear in the title bar perform the following steps.

Procedural Steps

| ## 6.1 | Click on Help -> Install Renesas IDE Features... |
| --- | --- |
| ## 6.2 | Select the Reality-AI features and click Finish. Restart e2studios when prompted. |
|  | You have finished this section. |

END OF LAB

THANK YOU