# Building a "Car State" AI Model in Minutes: The Renesas Explorer Tier Workshop

### Introduction:

In the world of IoT and asset tracking, knowing the precise state of a vehicle is a game-changer. Is a car "off"? Is it idling with the "AC on"? Or is it actively "driving"? Answering these simple questions with low-power sensors can unlock massive business value.

This is exactly what the Renesas Explorer tier trial workshop is all about. This hands-on guide walks you through building a machine learning model to detect these three car states using just an accelerometer sensor and the powerful Renesas AIK-RA6M3 board.

#### The Market Need for State Detection

Why is this simple classification so important?

- Fleet Management: Accurately track vehicle idle time (a major source of fuel waste) versus drive time. This optimizes fuel consumption, triggers predictive maintenance, and provides a clear picture of asset utilization.

- Usage-Based Insurance (UBI): Insurance providers can build far more accurate risk profiles by understanding not just if a car is moving, but its state.

- Car Sharing & Rentals: Automatically log when a vehicle is left idling versus when it's parked, improving billing accuracy and asset management.

### The Workflow: From Data to Deployed Model

The best part of this workshop is the simplicity and seamless integration of the toolchain. You don't need to be a data science expert to get a high-accuracy model.

The entire process is managed within the e2 studio IDE (and its Reality AI Utilities plugin) & Reality AI Tools:

- Collect Data: First, you use the Reality AI Utilities plugin inside e2 studio to configure the accelerometer on the AIK-RA6M3 board and collect data for your three classes (off, on w/ AC, driving).

- Train Model: Next, you send this labeled data to Reality AI Tools. This is where the magic happens. Its powerful AutoML pipeline automatically analyzes the complex sensor data, finds the best features, and trains an optimized machine learning model specifically for your microcontroller target.

- Deploy & Test: Finally, the fully trained model is imported back into e2 studio with a single click. You can then use the Reality AI Utilities again to deploy the model to your board and perform real-time testing and validation.

This seamless flow, from data collection to a fully integrated and tested model, all orchestrated from one IDE makes the development incredibly fast.

### The Environment: Tools & Hardware

This workshop is built on a powerful combination of Renesas hardware and Reality AI software.

#### Hardware

- Renesas AIK-RA6M3: This is our target hardware. It's a comprehensive development kit featuring the RA6M3 Arm® Cortex®-M4 microcontroller, along with a variety of sensors, including the accelerometer we use for this project.

  - Learn about the AIK-RA6M3 board

#### Software & Tools

- Renesas e2 studio: This is the core, Eclipse-based Integrated Development Environment (IDE) for all Renesas microcontrollers. It’s where you’ll manage your project, write code, and debug.

  - Learn about e2 studio

- Reality AI Utilities: This is an essential plugin for e2 studio. It acts as the bridge between your hardware and the Reality AI cloud. It provides the tools to configure sensors, capture and label data streams, and test the resulting AI models in real-time.

  - Learn about Reality AI Utilities

- Reality AI Tools: This is the cloud-based AutoML platform. You upload your raw sensor data, and it automatically generates and validates a tiny, efficient AI model that is optimized to run on the RA6M3. It completely removes the need for manual signal processing or deep learning expertise.

### Example Results

In our own experiments following this workshop, we achieved excellent results. The model's performance is best summarized by this confusion matrix:

As you can see, the Overall Accuracy is 97.9% and the F1 Score is 0.9787, which is a fantastic result for a three-class problem.

Let's break this down:

- The Diagonal (Blue): This shows all the correct predictions. The model correctly identified car_off 686 times, car_on_with_ac 689 times, and driving 689 times.

- Off-Diagonal (Errors): We see very few errors.

  - The model for driving (3) is exceptionally accurate, with a recall of 99.1% (bottom row, green). It was only mistaken for another state a handful of times. This makes sense, as driving motion is very distinct.

  - The most significant confusion (though still very low) is between car_off (1) and car_on_with_ac (2). This is logical, as the only difference is the subtle vibration from the AC system, which can be harder to detect than the motion of a moving car.

- Overall: The high precision and recall for all classes demonstrate that this is a robust and reliable model within the limited context of the experiments.

### Considerations for Your Own Development

While this workshop provides a fantastic result on our example data, there are two key things to remember when building your own models:

#### 1. Considerations for Evaluation

The data we used for this example was collected on a Prius hybrid. The vibration signature of a hybrid or a full-electric vehicle (EV) is very different from a traditional Internal Combustion Engine (ICE) car (e.g., the engine may not be running at all when the AC is on).

Because of this, customers will need to retrain their own model using data collected from their specific target vehicle(s) to achieve this level of accuracy in the field.

#### 2. Considerations for Own Development

This workshop is the perfect template. The best path forward is to take what you learn and apply it to your own hardware and data.

- Get the AIK-RA6M3 board.

- Follow the workshop to understand the flow.

- Collect data on your own car (or cars) in your target environment.

- Use the Reality AI Tools pipeline to generate and test your own custom model.

This process allows you to build a custom AI model for your specific needs in a fraction of the time it would normally take.

Would you like me to help you find more information on the Renesas RA6M3 microcontroller or other AI-based sensing applications?

<add some links for promotions>